SPECIFICATIONS 									Date

RE:	Name

Address

City, State Zip

(123) 456-7891

SCOPE OF WORK:

GENERAL NOTES:

All work done by the contractor must be in compliance with all state and local codes.

The contractor shall be responsible for obtaining all required licenses, certifications and permits, including, but not limited to, certification pursuant to the Federal lead-based paint activities program if applicable.  The contractor is also responsible for all inspections for all work related to this project.  Permit fees shall be included in the quote. All required permits must be obtained and copies provided to the Technologist Specialist at the time of billing. Failure to obtain and provide copies of the required permits will result in revocation of the service authorization, and in any event, no payment will be made until all required permits have been obtained and copies provided to ATP. If no permits are required, a signed statement to that effect should be provided at time of billing.

The contractor shall comply with all OSHA and Nebraska Department of Labor regulations.

The contractor shall notify the Assistive Technology Project (ATP) of any discrepancies between the drawings, specifications, existing conditions, and the applicable codes and regulations as soon as the discrepancies are identified.

To prevent issues of liability, the vendor/contractor must notify ATP of any problems with the condition of the home that could affect the modifications prior to starting work.  If prior notification is not given, it may be assumed that any damage is attributable to the contractor, unless otherwise documented/verified.

All products and equipment furnished and/or installed under the contract shall conform to the specifications.  No "Blemished", "Seconds", or reused building materials shall be used in this project unless otherwise noted.  Recycled or used equipment may be used provided it has been reconditioned to new and carry a warranty like new.  All recycled equipment shall be noted as such on the quote.  All specified items are to be provided and installed unless other wise noted.  All specified items are to be installed per manufacture’s instructions and in accordance with local code.

The contractor shall submit product literature on all "substitute" materials and receive approval by ATP staff before use or installation.  Submittal of product literature on listed materials is not necessary.

All material colors/finishes for materials including but not limited to paint, stain, flooring, tile, and grout, shall be selected by owner from full line of standard colors and finishes.

Work performed under the contract shall be executed by craftsmen skilled in the respective trades in a manner consistent with the standards of the respective trades, governing codes and generally accepted construction practices.

The contractor shall install all materials and equipment according to manufacturers' installation procedures and methods.  The contractor shall provide the consumer with instruction and written materials, including manufacturer's warranty, user's manuals and maintenance information.

The contractor is responsible for repair of all surfaces including furniture, walls, floor covering, doors, woodwork and trim, exterior pavement and yards, equipment, and fixtures affected during the course of construction to original or better condition.

The contractor shall keep the premises free from accumulation of waste materials or rubbish caused by construction activities and remove it from the premises on a daily basis.

The contractor shall warranty work, materials, and products for a minimum of one year.

The contractor shall refrain from any practice that is offensive or detrimental to the consumer's wellbeing while inside the consumer's home.

The project site must be inspected and measurements field verified before a quote is submitted.  The contractor will be liable for any extra costs incurred due to a lack of a reasonable visual observation of the project site.  Additional funds may be available to cover plan changes necessary due to conditions found that are not evident to a reasonable visual observation.  The contractor shall contact and receive ATP's written approval before changes in the plan or quote are made.

The contractor shall complete construction no later than 60 days from receipt of an authorization for the work.  If additional time is needed, the contractor shall submit a written request for approval by the property owner and ATP.  Failure to complete a project to the stated schedule may affect the contractor's ability to get future ATP contracts.

The contractor shall not solicit any products outside the range of work described in the specification and/or drawings when inspecting existing conditions (before submitting a quote) or after awarded an authorization.  Any solicitation may cause quote disqualification or cancellation of authorized work.

The contractor must maintain liability insurance for the duration of the project.  Proof of insurance may be required prior to issuance of the authorization for work.

The contractor shall have sole responsibility for any and all subcontractor's work.  All applicable terms and conditions of the contract shall bind subcontractors.

General Construction Note - Lumber:  All lumber used shall pressure treated, be SYP (Southern Yellow Pine) or Hem Fir, grade 2 or better with ACQ or MCQ treatment.  Lumber / plywood and pressure treated lumber shall conform to standard span tables.

General Construction Note - Plumbing:  Remove existing and salvage to consumer/dispose of per consumer’s instruction.  Provide and install plumbing fixtures as shown on plan.  Retrofit existing plumbing utilities and hook up to fixtures or locations. Plumbing installation, including but not limited to location of supply lines, drain lines and gas trap, shall be ADA compliant.  Coordinate mounting height and location of shower plumbing fixtures.  All shower wall penetrations shall be sealed with silicone sealant.  Toilet facilities and house water supply may be disconnected for no longer than 2/ 4/ 8 continuous hours without consumer’s permission.  Temporary toilet working condition and house water supply shall be restored before work can be stopped at the end of each working day unless authorized with prior written permission by the owner and ATP.  Install piping for hot and cold water and drain for shower and lavatory.  Install piping for cold water and wastewater removal for toilet.

General Construction Note - Electrical:  Provide and install or relocate and reinstall all electrical lighting, switches, outlets, and associated items as needed per local codes.  Consult with consumer/family for switch and outlet mounting heights unless otherwise noted. If circuit is being modified and required by local code, provide and install a Ground Fault Interrupter.

General Construction Note - HVAC:

Provide and install or relocate and reinstall all ventilation ductwork and associated items as needed per local codes.  Consult with consumer/family for locations unless otherwise noted.

New Ceiling/Walls, Repair, and Finish:

New walls shall be ½” drywall on exterior (both sides if needed) of 2x4 studs unless otherwise noted.  Walls/ceiling surfaces in bathroom shall be moisture resistant (green) drywall.  New surfaces directly in bathing area shall be concrete board.  Finish drywall joints with joint compound and needed joint accessories.  Sand all joint-finishing areas smooth and flush with drywall surface.  Finish all wall repair areas smooth, solid, and flush with surrounding areas as needed.  Patch walls and ceiling as required for areas disturbed during construction. Paint all new or newly exposed areas with 1 coat primer and 2 coats paint to match existing – coordinate with consumer/family.

Patching and Painting:  Patch walls and ceiling as required for areas disturbed during construction using like materials to match the existing.  If necessary, paint walls and ceiling with primer and apply two coats of latex paint. Color coordinated with owner.

Interior Door: 32”/ 36” wide, standard height, interior grade hollow core door.  Finish trim and door to match existing.  Provide and install new interior grade locking lever hardware.

"Swing Clear" Hinges (Interior Doors): Interior grade regular duty swing clear hinges.  Hinges must allow door to completely clear the door opening when open 90 degrees.  Hinges to match existing.

Swing Clear Hinges:  Accessible Environments (www.accessibleenvironments.net or 800-776-1461) or Lowes.com (http://www.lowes.com/Offset-Hinges/_/N-1z10lys/pl) or Hardware Source (http://www.hardwaresource.com/hinges/door-hinges/wide-throw-hinges-swing-clear-hinges or 877-944-6437) or approved substitute.

Accordion Door:  Economy grade 24”/ 32”/36”/ 48” accordion door.  Provide and install wood trim to match existing as needed.  Finish door and trim as needed to match existing.  New doors may be wooden or vinyl.  Provide and install track, doorknobs and locking hardware (Folding Lock).

Closet Bi-fold Door: New economy grade 2’-0”/ 2’-6”/ 2’-8”/ 3’-0” x 6’-8” bi-fold closet doors. Provide and install wood trim to match existing as needed.  Finish door and trim as needed to match existing.  New doors to be finished (color per consumer) and to have louvers located in the doors. Provide and install necessary track, doorknobs and hardware.

Pocket Sliding Door:  New economy grade 2’-8/3'-0" x 6'-8” sliding door and track system. Door style to match existing. Door and track system to be mounted within confines of the wall.  Provide and install wood trim to match existing as needed.  Finish door and trim as needed to match existing.  Hardware to include "D" shaped pulls on both sides of the door and limit stops as needed.  Install lock appropriate for pocket door.

Sliding Door:  New economy grade 3'-0" x 6'-8" hollow core wood sliding door and track system.  Door and track system to be mounted on the exterior of the doorway; verify with consumer.  Finish door as needed to match existing.  Hardware to include "D" shaped pulls on both sides of the door and limit stops as needed.

Wall Mounted Door:  Johnson wall mounted door hardware set found at (homedepot.com) Model # 2610F721, or approved substitute.

Sub-Floor, Flooring, & Baseboard:  Inspect sub-flooring to insure safety and stability.  Patch and repair floor as needed.  Replace entire bathroom with 3/4” CDX plywood if necessary. Provide and install under-layment approved by vinyl manufacturer.

Medium grade vinyl seamless flooring.  Coordinate flooring style and color with owner.  Wood stained/painted baseboards are to match existing.

Carpet: Commercial grade solution dyed nylon loop carpet with a pile thickness of ½” (maximum) with no pad.  Carpet shall be no less than 26 oz. minimum.  Economy grade carpet shall be no less than 20 oz. minimum.   Reducer shall be used between carpet and vinyl floors. Wood painted or stained baseboards are to match existing.  Coordinate style and finish with consumer/family.

Grab Bars: Stainless steel concealed flange, 1 ¼”/ 1 ½” diameter ADA compliant grab bars with non-slip surface. Coordinate with consumer/family on final position/location of each grab bar.  Seal all areas as needed to avoid water penetration into wall.  Lengths are shown on drawing.

Grab Bar Blocking:  (Standard) Provide blocking as needed to mount as shown on plan. Blocking shall consist of 2x6 boards mechanically fastened between studs behind drywall/cement board and grab bar mounting point or other wall finish or 3/4" thick x 6" wide (min.) plywood recessed and mechanically fastened into the studs under drywall/cement board extending the entire length of the proposed grab bar unless otherwise noted.

Grab Bars: (WingIts or Moen Grab Bar Securement Kit) Install grab bar in studs in wall if possible, if not, provide and install securement kit per manufacturer’s recommendations in walls with thickness of minimum 5/8”.  Coordinate with consumer/family on position/location of grab bar.

Grab Bars WingIts:  WingIts Grab Bar Fastener Stainless Steel Standard (Item # GBW40) at www.wingits.com or 877-894-6448; WingIts Grab Bar Fastener Standard Stainless Steel Beast with 4 inch screw (Item #202670710) at www.homedepot.com; Grab Bar WingIts (Item #GBW40) at www.grabbarspecialists.com or 800-707-5033) or approved substitute.

Moen Grab Bar Secure Mount Wall Anchors:  Moen SecureMount, at www.moen.com/securemount; Moen Home Care SecureMount Stainless Steel Grab Bar Anchors in Chrome (Model #SMA1000CH or Internet Model #202044138) at www.homedepot.com or 800-466-3377 or approved substitute.

Grab Bars: (Solid Mount) Provide blocking as needed to mount as shown on plan. For areas in bathtub/shower where the shower panels may have a cavity between panel and studs/cement board provide additional blocking as needed. Coordinate with consumer/family on final position/location of each grab bar.  Seal all areas as needed to avoid water penetration into wall.

Grab Bars Solid Mount: The Solid Mount, at www.grabbarspecialists or 800-707-5033; Solid Mount Grab Bar Mounting Kit, at www.grab-bar.com or 877-643-8468 or approved substitute.

Grab Bars: (Exterior Wall Mount) Install grab bar in stud in wall if possible.  If not, provide blocking as needed to secure safely.  Blocking shall consist of a 1 x 6/2 x 6 board with a 1/4” radius edge sanded and finished to match existing trim/wall surfaces in bathroom, secured to studs in wall with at least 2 countersunk screws per stud (minimum 4 screws per 1 x 6/2 x 6 board).  Spanning appropriate distance.  Use wood putty or plugs to conceal screws.  Coordinate with consumer/family on finish of 1 x 6/2 x 6 board and final location of grab bar.

Extend a Hand Grab Bar:  When raised, this handy bar extends 12” from the wall to place within reach.  Bar locks automatically when raised.  Can be left permanently in position, or lowered by pushing two release buttons.  600 lb weight capacity.

Extend a Hand Grab Bar:  Model #920325/26, at pattersonmedical.com or 800-323-5547

The Extend a Hand Grab Bar™.

Grab Bar at Toilet: Stainless steel, concealed flange, 1 1/2" diameter ADA compliant straddle bar for tank toilet/floor mounted straddle bar/wall to floor bar/urinal bar with non-slip surface. Coordinate with consumer/family on final position/location of each grab bar.

Straddle Bar For Tank Toilet: Model #2424-SN, at www.calltsm.com or 800-472-2227; Model #64, at www.adaptiveaccess.com or 281-856-9332 or approved substitute.

Floor Mounted Straddle Bar: Model #2228-S, at www.calltsm.com or 800-472-2227; Model #49, at www.adaptiveaccess.com or 281-856-9332 or approved substitute.

Wall to Floor Bar: Model #3033 (with outrigger, Model #3033WX), at www.calltsm.com or 800-472-2227; Model #14-1 (with outrigger, Model #15/16), at www.calltsm.com or 800-472-2227 or approved substitute.

Floor Mounted Hinged Support Rail:  Provide and install a floor mounted hinged arm support rail that automatically locks in the upright position. Unit to be stainless steel or powder-coated.  Provide blocking as needed in floor and install according to manufacturer’s installation instructions.  Coordinate position of device with consumer/family prior to installation to ensure appropriate use.  Provide and install any miscellaneous fasteners as required for secure installation.  The hinged arm support should have a clamp on toilet paper holder if necessary.

Floor-Mounted Folding Rail: Devon Floor-Mounted Folding Support Rail (Model #AA2026)  (toilet paper holder sold separately Model #557476), at www.pattersonmedical.com or 800-323-5547 or 800-343-9742; Linido Hinged Arm Support Floor Console (optional single handle Toilet Roll Holder), at www.rehabmart.com/product/hinged-arm-support-36105.html or 800-827-8283 or approved substitute.

P.T. Rail with Hinged Floor Mast: P.T. Rail with Hinged Floor Mast (toilet paper holder sold separately), at www.healthcraftproducts.com, 888-224-3239 or approved substitute.

Wall Mounted Hinged Support Rail: A stainless steel or powder-coated wall mounted hinged arm support rail that automatically locks in the upright position. Provide blocking as needed in wall (minimum 2 x 12 blocking).  Coordinate position of device including height with consumer/family prior to installation to ensure appropriate use.  Provide and install any miscellaneous fasteners as required for secure installation.  The hinged arm support should have a clamp on toilet paper holder if necessary.

Wall Mounted Hinged Support Rail:  Devon Deluxe Support Rail (Model #AA2012) (toilet paper holder sold separately, Model #557476), Devon Support Rail with Folding Leg (Model  AA2018), Etac Optima Toilet Armrests with Support Leg (Model #557403) at www.pattersonmedical.com or 800-323-5547 or 800-343-9742; Linido Hinged Arm Support #(optional single handle Toilet Roll Holder), (Model #36105) at www.rehabmart.com or 800-827-8283; Moen Secure Mount Glacier 30” Flip Up Grab Bar (Item #R8962FDW), at www.moen.com or 800-289-6636 or approved substitute.  Moen products also sold at participating Lowe’s, Home Depot and Menards.   Hinged P.T. Rail (Right/Left) (toilet paper holder sold separately) or Dependa Bar, at www.healthcraftproducts.com or 888-619-9992; Tubular Specialties Manufacturing, Swing-up Bar (Model #3012) (Toilet Paper Dispenser, Model #3012 PD catalog pg. 19), at www.calltsm.com or 800-472-2227 or approved substitute.

Advantage Rail:  Provide and install an Advantage Rail as noted on plans in floor according to manufacturer’s installation instructions.  Provide blocking as necessary.  Provide and install any miscellaneous fasteners as required for secure installation.  Coordinate position of device with consumer/family prior to installation.

Advantage Rail:  Model #AR-T (Up to 300lbs), Model AR-T-HD (Up to 450lbs), at www.healthcraftproducts.com or 877-224-3239; Model #554955, at www.pattersonmedical.com or 800-323-5547 or 800-343-9742 or approved substitute.

SuperPole:  Install  SuperPole, as noted on plans in floor and ceiling according to manufacturer’s installation instructions.  Provide blocking as necessary.  Provide and install any miscellaneous fasteners as required for secure installation.  Coordinate position of device with consumer/family prior to installation. (For 45 to 90 degree ceilings include Angled Ceiling Bracket, Model #565408, at www.pattersonmedical.com, (For ceiling heights greater than 93’ - 99” include UniFit Extension Tube, Model #552673, at www.pattersonmedical.com. SuperPole Only (up to 300 lbs.):  SuperPole (Model # 552668), at www.pattersonmedical.com or 800-323-5547; SuperPole, at www.healthcraftproducts.com or 877-224-3239 or approved substitute.

SuperPole with SuperBar: Model # 552670, at www.pattersonmedical.com or 800-323-5547 or SuperPole with SuperBar, at www.healthcraftproducts.com/superbar.htm or 877-224-3239 or approved substitute.

SuperPole HD (up to 450 lbs.): Model # 552669, at www.pattersonmedical.com or 800-323-5547 or SuperPole with SuperBar, at (www.healthcraftproducts.com/superbar.htm or 877-224-3239 or approved substitute.

SuperPole with SuperTrapeze: Model #552672, at www.pattersonmedical.com or 800-323-5547 or SuperPole with SuperBar, at www.healthcraftproducts.com/superbar.htm or 877-224-3239 or approved substitute.

Toilet: (Existing) Reuse existing toilet.  Relocate as shown on drawing.  Seal as necessary.

Toilet: (New Standard) Install a new ADA compliant low water consumption elongated bowl (18 1/2"x15")/standard bowl (16 1/2"x15") toilet with a rim height of 17 1/4"-/19" AFF as shown on plan.  Toilet shall have a minimum 3” flush valve.  Seal as necessary and provide and install a closed front toilet seat with lid.

Toilet:  Kohler Highline Classic the Complete Solution 2-piece elongated toilet; Kohler Santa Rosa Comfort Height, one piece elongated toilet; Kohler Cimarron Comfort Height, two piece elongated toilet; Kohler Bancroft ADA (has a bidet option), at www.kohler.com; Saratoga Relaxed Height Elongated Toilet www.craneplumbing.com or approved substitute.  American Standard Optum Vor Max Complete Right Height 2-piece Elongated toilet at homedepot.com or approved substitute.

Toilet: (Elongated-Space Saver - fits the space of a conventional round bowl) Install ADA compliant low water consumption compact elongated front toilet with a rim height of 16 ½” AFF and 26 ½” from wall. Toilet shall have a minimum 3” flush valve.  Seal as necessary and provide and install a closed front toilet seat with lid. Toilet: American Standard Saver Right Height Elongated, one piece compact toilet; American Standard Watersense high-efficiency, one piece toilet, at www.americanstandard-us.com or 800-442-1902; Kohler Santa Rosa Comfort Height, one piece compact elongated, at www.kohler.com or approved substitute.

Toilet: (Bariatric) Provide and install a new ADA compliant low water consumption elongated bariatric toilet with a rim height of 17” min. AFF.  Toilet shall have a minimum 3” flush valve. Install per manufacturer’s recommendations.  Seal as necessary.  Toilet lid included. Bariatric Toilet: Zurn Wall Hung Extra Heavy Duty 1000 lb. Bariatric Series (Item # Z5690), at www.zurn.com and Whitehall (with Big John Toilet Seat), Floor (2125A-W-3-EG10-HSBJ) or Wall Mounted (2125A-W-2-EG10-HSBJ), at www.whitehall.com or approved substitute.

Big John Toilet Seat:  Regular Seat (2 1/2” Rise and 1200 lbs. weight capacity) or Standard Seat (1 1/2” Rise and 800 lbs. Capacity), at www.bigjohntoiletseat.com or www.alimed.com or approved substitute.

Wall Mounted Toilet with Supports:  Increases weight capacity of a wall-hung toilet to 1000 lbs. capacity. Wall-Mounted-Toilet Support, Standard (adjust 7 1/2” -14”, Item # 710145) or Short Swivel Toilet Support (adjusts 5” to 7 1/2”, Item # 711635), at www.alimed.com or Toilet Supports, at www.bigjohntoiletseat.com or approved substitute.

Wall Hung Toilet: Install a new ADA compliant low water consumption elongated wall hung toilet with a rim height of 17” min. AFF.   Install per manufacturer’s recommendations.  Seal as necessary.  Toilet lid included.

Wall Hung Toilet: Kohler Kingston Wall Mounted with rear inlet (K-4323, Elongated Bowl) or (K4323-L, Elongated Bowl with Bedpan Lugs), at www.kohler.com or approved substitute.

Toilet Supports:  Increases toilet to 1000 lbs. weight capacity. High-impact ABS leg with flexible, stain/chemical resistant PVC carriage that conforms to the contour of the bowl. Fits under most wall-mounted toilets.  Height adjusts by screwing/unscrewing leg, 7 1/2” to 14” for Standard Legs and 5” to 7 1/2” for short swivel. Wall-Mounted-Toilet Support, Standard (adjust 7 1/2” -14”, Item # 710145) or Short Swivel Toilet Support (adjusts 5” to 7 1/2”, Item # 711635), at www.alimed.com or Toilet Supports, at www.bigjohntoiletseat.com or approved substitute.

Toilet Supports

Toilet: (Corner) Install a new corner toilet. The toilet will have a standard 1.6-gallon flush with a rim height of 16 1/8 -17” AFF.  Toilet shall have a minimum 3” flush valve. Install as per manufacturer’s recommendation as shown on plan.  Seal as necessary and provide and install a closed front toilet seat with lid.

Corner Toilet:  Barnum Dual Flush Corner Toilet with seat, at www.signaturehardward.com or 866-855-2284 or approved substitute; Triangle Cadet 3 Right height toilet with seat (Elongated-Model# 270AD.001/ Round-Model# 270BD001) or, at www.americanstandard-us.com or 800-442-1902 or approved substitute.

Toilet Riser:  Install a 3 1/2" toilet raiser (installed between finished floor and base of toilet) with plumbing extenders as needed and existing toilet. Install toilet raiser per manufacturer's recommendation and seal as necessary.Toilevator: Model #6793, at www.pattersonmedical.com or 800-323-5547; Toilevator, at www.hartmobility.com or 877-760-5415; Toilevator (Model #AH100) or Toilevator Grande Extra Long (Model #AH350), at www.clarkehealthcare.com or 888-347-4537; Toilet Elevator (Model #TE1), at www.grabbarspecialists.com or 800-707-5033 or approved substitute.

Bidet: Install a bidet with warm water, dryer and remote control at existing elongated/round bowl toilet.  Item will require a GFIC outlet field verify for best location next to toilet. Install per local codes.

Bidet:  Toto Washlett S300e Bidet (Purchase with appropriate lid), Elongated (SW574T20), Round (SW573#01), at www.totousa.com or 888-295-8134 or Toto Washlet S350e Bidet (Purchase with appropriate lid, Elongated (SW584#01), Round (SW583#01), at www.totousa.com or 888-295-8134; Infinity Bidet for one or two piece toilets, Elongated with Remote (081564475) or Round with Remote (081564467), at www.pattersonmedical.com or 800-323-5547; Kohler C3-125, Elongated Bowl Bidet with Side Controls (K4737-0), at www.kohler.com or approved substitute.

Bariatric Bidet: Provide and install a bidet with warm water, dryer and remote control at existing elongated toilet.  400 lb. weight capacity.  BB 2000 Elongated Bidet Toilet Seat with Remote (081621655), at www.pattersonmedical.com or 800-323-5547

BB 2000 Elongated Bidet Toilet Seat

Lavatory Faucet: (Single Lever Handle) Install a light commercial grade ADA compliant single lever handle pressure-balanced lavatory faucet with temperature limit stops, pop-up waste assembly and 4" centers, plumbing supplies and accessories as needed.  Install faucet and plumbing including hot and cold water supply and drain per codes and manufacturer’s recommendations.  Faucet: Moen Chateau (Model # L4621), at www.moen.com or 800-289-6636; American Standard Colony Soft Single Control (Model # 2175.506), at www.americanstandard-us.com or 800-442-1902; Gerber Single Handle Lavatory (Model # 40-145-VP), at www.gerberonline.com or approved substitute.

Lavatory Faucet: (Two Handle) Provide a light commercial grade ADA compliant two handle pressure-balanced lavatory faucet with temperature limit stops, pop-up waste assembly and 4"/8” centers, plumbing supplies and accessories as needed.  Install faucet and plumbing including hot and cold water supply and drain per codes and manufacturer’s recommendations.  Faucet: Moen Chateau (Model #4925), at www.moen.com or 800-289-6636; American Standard Colony Soft Single Control (4” center) (Model # 2275.500), American Standard Colony Soft Single Control (8” center) (Model #3875.501), at www.americanstandard-us.com or 800-442-1902, or approved substitute.

Lavatory: (Pedestal) Install a new lavatory and pedestal unit. Provide wall blocking as needed to mount lavatory and install as per manufacturer’s recommendations. Lavatory and Pedestal: Gerber Maxwell (Model # 22-504, 4” Centers, measures 20 1/2” W x 17 3/4” D x 33 3/4” H), at www.gerberonline.com or 888-648-6466; American Standard Cadet 24” Pedestal Sink (Model #0236411, 4” centers), (Model #0236811, 8” Centers) 24-1/2” x 20” x 35”, or Cornice (Model #0611.400, 4” Centers),  (Model #0611.100, Center Hole Only, measures 15 ½’ x 15 ½” x 31”) at www.americanstandard-us.com or 800-442-1902 or approved substitute.

Gerber Maxwell		American Standard Cadet	           American Standard Cornice

Lavatory: (Euro With Base Cabinet) Install a Euro-style base cabinet/lavatory.  Install as per manufacturer’s recommendations.

Euro Lavatory & Base Cabinet: Style Selections 25” Euro Style Single Sink Bathroom Vanity with Sink (Item #648775 White/ #648776 Espresso), 25”W x 18.6”D x 33”H, 4” on Center, or Style Selections 19” Euro Style Single Sink Bathroom Vanity with sink (Item # 648774 White), or Style Selections 31” Euro Style Single Sink Bathroom Vanity with Sink (Item #648777 Espresso) (Faucet not included), at www.lowes.com; 36” Magicwoods Eurostone Vanity Combo (Item #6059036, 36”W x 13-1/4”D x 34”H, 4” on Center, Faucet not included), at www.menards.com or approved substitute.

19” Style 			25” Style				36” Magickwoods Euro

Selections Euro		Selections Euro

Wall Hung Roll Under Lavatory:  Provide and install an ADA compliant wall hung lavatory with backsplash.  Provide blocking as necessary to mount on wall and install per manufacturer’s recommendations. Lavatory shall be mounted @ 34" AFF/ Coordinate with consumer/family on height of lavatory.  For corner lavatory, mount small counter tops on both sides of the lavatory. See drawing for dimensions.  Lavatory (Wheelchair - ADA):  Gerber Eaton Wheelchair (Model #12-464, 20”L x 27”W), at www.gerberonline.com or 866-538-5536); American Standard Wheelchair White Bath Sink (Model # 9141.011.020, 27”L x 20”W), at www.lowes.com; Mansfield Wheelchair ADA, Wall-Mount Lavatory (Model# 315410040, 20”L x 27”W), at www.menards.com or approved substitute.

Lavatory (Standard):  Kohler Kingston (Model #2005-0, 21 1/4”L x 18 1/8”W), at www.kohler.com; Gerber Monticello II (Model #12-654, 20 1/4"L x 181/2”W), or Gerber Hayes (Model #12-384, 20”L x 18”W), at www.gerberonline.com or 866-538-5536) or approved substitute.

Lavatory (Corner): Kohler Marston (Model #K-2766-EH-0, 16” L x 16” W), at www.kohler.com; Elizabethan Classics English Turn Corner Basin Sink (Model #ECETCBWHSP-S, 14” L x 14” W), at Vintage Tub & Bath, www.vintagetub.com or 877-796-3264 or English Turn Wall Hung Corner Sink White (Model # IMC-150WB, Internet # 100026924), at www.homedepot.com or approved substitute.

English Turn Wall-Hung

Lavatory (with sides): Whitehaus Wall Mount Sink (Model #WH103SP40, Specify R or L, 17 1/2” L x 8 1/2” D x 5 1/2”H), at www.vintagetub.com or 877-868-1369 or approved substitute.

Lavatory (Space Saver): Jellbeck Porcelain wall-mount sink (Model # 932627, 14” L x 12”W), at www.signaturehardware.com or 866-855-2284; Gerber West Point Space Saver Wall Hung (Model # 12-354, 14” x 12”), at www.gerberonline.com or approved substitute.

Counter Mounted Roll Under Sink:  Provide and install an ADA compliant drop-in (counter mounted) lavatory w/ 4" centers.  Countertop shall be mounted @ 34" AFF/ Coordinate with consumer/family on height of countertop.  See the plan for location and length of countertop.   Install blocking as needed to support countertop.  For installation between walls support countertop with 2x4s secured to the walls at the counter perimeter. Lavatory:  Kohler Ellington (Model #K- 2906, 19 1/4”L x 14 1/4”W), at www.us.kohler.com or 800-456-4537; Tierra Drop-In Bathroom Sink (Model #85400, 18”L x 22-3/4” W), at www.homedepot.com; Gerber Maxwell Self Rimming (Model #12-88-CH, 19” L x 19”W), at www.gerberonline.com or approved substitute.

Countertop: 20" D x 1 1/2" thick plastic laminate covered counter top with a 4" x 3/4" plastic laminate covered backsplash.  See plan for length.

Leg Protection: (Plywood Shroud) Pipe leg protection shall consist of a 1/2" plywood shroud with 2x4 frame angling back from under the front edge of the countertop to the back wall 9" above the floor.  The shroud shall have a 18"x 20" plumbing access hatch, secure with 2 countersunk screws on each side and plywood closed ends (if they exist.)  The shroud shall be sanded and smooth to the touch.  Sand and round any sharp corners. Coordinate with owner for shroud finish.

Leg Protection:  (Pipe Guard) Install a protective under sink pipe covers/plumbing enclosure.  Install per manufacturer’s recommendations.

Pipe Cover:  IPS/Truebro, (Lav Guard 2 or Soft Guard Plus), at www.truebro.com or 800-888-8312); Grab-Bar.com (Trap Wrap Cover kit for Strainer, Stops and Trap, SKU#K503KN), at www.grab-bar.com or 800-643-8498) or approved substitute.

Plumbing Enclosure: IPS/Truebro (Lav Shield), at www.truebro.com or 800-888-8312 or approved substitute.

Prefabricated Fiberglass Barrier-Free Shower Floor (Surface Mount): Install a barrier free shower floor (verify with plan for location of drain) and shower threshold, (see Shower Threshold) and 2" stainless steel drain.  Mounting floor shall be made level for proper shower drainage.  Reinforce joists as needed under shower mounting area to maintain level. Secure overlapping vinyl waterproof pan (if required by code) on clear and clean floor under shower floor and up adjacent walls 9” (if required by code) Install and secure shower floor per manufacturer's recommendation.  Install shower threshold at shower entry.  Apply silicone sealant as needed at all edges, corners, and material changes for watertight fit. Install "Shower Wall System" as specified on walls over shower base and vinyl pan overlapping (if exists).  Shower Floor:  E.L. Mustee & Sons, Inc. (Durabase, Caregiver 360 Barrier Free Shower Floor, 60”L x 30”W or Durabase, Caregiver 3865 Barrier Free Shower Floor, 65” L x 38”W), at www.mustee.com or 800-321-3128 or approved substitute. E.L. Mustee products are also available on special order at participating Home Depot or Menards.

Shower Ramp: E.L. Mustee & Sons, Inc. (Durabase 360.100, 60”L x 12”D), at www.mustee.com or 800-321-3128 or approved substitute.

Prefabricated Fiberglass Barrier-Free Shower Floor (Recessed): Install barrier free shower floor (verify with plan for location of drain) or approved substitute with shower threshold (see Shower Threshold), and 2" stainless steel drain. Prepare floor as needed for proper installation and drainage. Recess into floor so finished shower floor is no greater that 1/4" above floor level/flush with floor level.   Reinforce joists as needed.  Shower sub-floor shall be 3/4" (min) thick CDX plywood securely fastened with nails/screws or other mechanical fasteners to joists and cross blocking as needed.  Reinforce joists with 3/4” CDX exterior plywood gussets on both sides of the notched floor joist as needed under shower mounting area to maintain level.  Glue and nail gusset to floor joist – nail at a maximum of 6” between nails.  Extend gussets as far under existing floor as possible.  Provide a sister joist between existing floor joist as needed.  Installation of sub-floor shall be level to insure proper shower floor drainage.  Secure overlapping vinyl waterproof pan over sub-floor and up adjacent walls 9" (if required by code). Install and secure shower floor per manufacturer's recommendation.  Install shower threshold at shower entry.  Apply silicone sealant as needed at all edges, corners, and material changes for watertight fit.  Provide and install "Shower Wall System" on walls over shower base and vinyl pan overlapping (if exists).

Shower Floor:  E.L. Mustee & Sons, Inc. (Durabase, Caregiver 360 Barrier Free Shower Floor, 60”L x 30”W or Durabase, Caregiver 3865 Barrier Free Shower Floor, 65” L x 38”W), at www.mustee.com or 800-321-3128 or approved substitute. E.L. Mustee products are also available on special order at participating Home Depot or Menards.

Cultured Marble Shower Floor (Surface Mounted): Install a surface mounted cultured marble ADA compliant barrier free shower floor (verify with plan for location of drain) with entry ramp and shower threshold, (see Shower Threshold).  Provide and install a 2" stainless steel drain.  See plan for shower floor size.  Shower floor shall have minimum edge thickness of 1 3/8” with minimum thickness of ½” at drain.  Mounting floor shall be made level for proper shower drainage.  Reinforce joists as needed under shower mounting area to maintain level. Secure overlapping vinyl waterproof pan on clear and clean sub-floor and under shower floor and up adjacent walls 9" (if required by code).  Install and secure shower floor per manufacturer's recommendation.  Install shower threshold at shower entry.  Apply silicone sealant as needed at all edges, corners, and material changes for watertight fit. Install "Shower Wall System" on walls over shower base and vinyl pan overlapping (if exists).Shower Floor: Roman Marble Products, Inc., at www.counterstone.org or 800-347-6699; Creative Marblecast Mfg., 605-399-9648; Onyx Collection, (Low Profile Base with optional ramp; Standard Bases, (Field Verify for Size), at www.onyxcollection.com or 800-669-9867 or approved substitute.

Shower Ramp: E.L. Mustee & Sons, Inc. (Durabase 360.100, 60”L x 12”D), at www.mustee.com or 800-321-3128; Onyx Collection, Ramped Curb, (Field Verify for Size), at www.onyxcollection.com or1-800-669-9867 or approved substitute.

Cultured Marble Shower Floor (Recessed): Install a recessed cultured marble ADA compliant barrier free shower floor with shower threshold (see Shower Threshold) and 2" stainless steel drain. See plan for shower floor size.  Recess into floor so finished shower floor is no greater that 1/4" above floor level/flush with floor level.   Reinforce joists as needed.  Shower subfloor shall be 3/4" (min.) thick CDX plywood securely fastened with nails/screws or other mechanical fasteners to joists and cross blocking as needed.  Reinforce joists with 3/4” CDX exterior plywood gussets on both sides of the notched floor joist as needed under shower mounting area to maintain level.  Glue and nail gusset to floor joist – nail at a maximum of 6” between nails.  Extend gussets as far under existing floor as possible.  Provide a sister joist between existing floor joist as needed.  Installation of subfloor shall be level to insure proper shower floor drainage.  Secure overlapping vinyl waterproof pan over subfloor and up adjacent walls 9" (if required by code). Install and secure shower floor per manufacturer's recommendation.  Install shower threshold at shower entry.  Apply silicone sealant as needed at all edges, corners, and material changes for watertight fit.  Provide and install "Shower Wall System" on walls over shower base and vinyl pan overlapping (if exists). Shower Floor: Roman Marble Products, Inc., at www.counterstone.org or 800-347-6699; Creative Marblecast Mfg., 605-399-9648; Onyx Collection, (Low Profile Base with optional ramp; Standard Bases, (Field Verify For Size), at www.onyxcollection.com or 800-669-9867 or approved substitute.

Shower Wall System - Cultured Marble Shower Panels Over Cement Board: Install watertight shower wall system up to 70" A.F.F. min. (unless otherwise noted).  Walls directly adjacent to shower (see plan) shall receive a 3/8” cultured marble shower paneling system adhered per manufacturer's recommendation to ½” cement board, of thickness to match existing walls for flush installation, securely fastened with nails/screws or other mechanical fasteners appropriate for cement board to wall studs.  Vertical joints of paneling and cement board shall be staggered to avoid penetration of leaking water through system.  Thoroughly seal paneling at edges to prevent water penetration.  Provide and install edge/corner/cap accessories and sealant as needed at all joints and edges, and top of watertight shower wall panels.  Exposed wall surface above shower wall system shall be moisture resistant (green) drywall. - see “Wall Repair and Wall Finish" for finish.

Shower Panels: Roman Marble Products, Inc., at www.counterstone.org or 800-347-6699; Creative Marblecast Mfg., 605-399-9648; Onyx Collection, (Low Profile Base with optional ramp; Standard Bases (Field Verify Size), at www.onyxcollection.com or 800-669-9867 or approved substitute.

Shower Wall System – Fiberglass/Thermoplastic Shower Panels Over Cement Board:

Install watertight shower wall system up to 70" A.F.F. minimum (unless otherwise noted) with continuous grab bar blocking.  Walls directly adjacent to bathtub/shower shall receive a 3 or 5 piece fiberglass/thermoplastic shower paneling system adhered per manufacturer's recommendation to ½” cement board securely fastened with nails/screws or other mechanical fasteners to wall studs.  Shower wall system shall include a minimum of 2 soap/shampoo shelves integral to the shower panels.  Vertical joints of paneling and cement board shall be staggered to avoid penetration of leaking water through system. Thoroughly seal paneling at edges to prevent water penetration.  Provide and install edge/corner/cap accessories and sealant as needed at all joints and edges, and top of watertight shower wall panels.  Exposed wall surface above shower wall system shall be moisture resistant (green) drywall of thickness to match existing walls for flush installation, see "Wall Repair and Wall Finish" for finish.  No Plastex, FRP, or similar material is allowed as a substitute for this project.

Bathtub Panels: E. L. Mustee & Sons, Inc., (Fiberglass, Model #350 (w/out shelving) or Fiberglass, Model #557  (w/shelving)), at www.mustee.com or 216-267-3100, or approved substitute.

Shower Panels: E.L. Mustee & Sons, Inc., (Thermoplastic, Model #260, at www.mustee.com or 216-267-3100) or approved substitute.

Window Trim Kit: Provide and install a fiberglass window trim kit to avoid water penetration around window in bathtub/shower area.  Field verify window dimensions prior to ordering.  Install and seal per manufacturer’s recommendations. Window Trim Kit: Shower Floor:  Duratrim Fiberglass, (Model #300.600) or Duratrim Thermoplastic, (Model #52.600W), at www.mustee.com or 216-267-3100; Swan Corporation Window Trim Kit, (Model #WTK-1), at www.swanstone.com or 800-325-7008 or approved substitute.

Tile Shower Floor & Entry Ramp:  Install ceramic tile surfaced ADA compliant shower floor with vinyl shower liner, 2" steel drain and shower threshold  (see "Shower Threshold").  See plan for shower floor size.  Recess into floor so finished shower floor is no greater that 1/4" above floor level. Shower subfloor shall be 3/4" (min.) thick CDX plywood securely fastened with nails/screws or other mechanical fasteners to joists and cross blocking as needed. Installation of subfloor shall be level to insure proper shower floor drainage.  Using 3:1 sand/cement mix pour pre-slope layer over subfloor with outside edge thickness at least 1 ½” sloped to drain.  Secure overlapping vinyl waterproof pan over this layer and up adjacent walls 9".  Pour second layer of sand/concrete mix over liner and slope to drain at 1/4" per foot max. – outside edge of the sand/concrete layer shall be a minimum of 1" thick.  Slope entry ramp using sand/concrete mix (see plan for dimensions).  Install 2"x2"slip resistant ceramic tile in mortar bed over layer of sand/concrete mix.  Grout with anti-mold modified grout.  Seal grout lines 3 times according to manufacturer’s instructions.  Install shower threshold at entry to shower.  Apply sealant as needed at all edges, corners and material changes.  Provide and install "Shower Wall System" on walls over shower base and vinyl pan overlapping.

Shower Wall System:  (Tile Shower Walls) Install watertight 4”x4”/12”x12” tile wall system up to 6'-0" AFF with continuous grab bar blocking zone. Walls directly adjacent to bathtub or shower shall receive tile surface over 1/2" concrete board securely fastened with screws or other mechanical fasteners appropriate for materials to studs.  Provide and install tile as per manufacturer's recommendation. Grout with anti-mold modified grout.  Seal grout lines 3 times according to manufacturer’s instructions.  Apply sealant as needed at all edges, corners and material changes.  Seal (caulk) joint between tile wall and painted wall as needed.  If installing grab bars into tiled surface, drill through tile with a tile bit and securely mount to existing wall stud or grab bar blocking.  Exposed wall surface above shower wall system shall be moisture resistant (green) drywall, (see “Wall Repair and Wall Finish" for finish). Blocked-in Window:  Remove window trim and window down to rough opening.  Infill opening with structure to match existing construction.  Insulate and apply interior wall finish to match existing.  Apply siding on exterior to match existing.  Seal joints between existing and new siding with silicone sealant.  Paint with primer and two coats paint to match existing.

Shower Threshold: Install a 1 3/8” tall roll-over Collapsible Water Retainer at shower entry, or approved substitute.  Apply and seal per manufacturer’s recommendations.  See plan for length of threshold.

Shower Threshold: The Waterstopper, (Model #TWS), at Best Bath Systems, Inc., www.best-bath.com or 800-727-9907 or approved substitute.

Shower Threshold:  Install a 1 1/2” tall 1” wide roll-over collapsible water retainer at shower entry.  Apply and seal per manufacturer’s recommendations.  See plan for length of threshold.

Shower Threshold: Collapsible Water Retainer, at www.allinonemobility-shop.com or 800-944-4935; Collapsible Water Retainer, at Accessible Design and Consulting, www.accessibleconstruction.com or 310-215-3332 or approved substitute.

Shower Threshold:  (E.L. Mustee) Install a 1" tall roll-over drip rail at shower entry. Apply and seal per manufacturer’s recommendations.  See plan for length of threshold. Shower Threshold:  Drip Rail (Model# 360.200), at E.L. Mustee & Sons, Inc., www.mustee.com or 216-267-3100 or approved substitute.

Shower Threshold:  (Cultured Marble)  Install a ¾” tall " x 3” wide cultured marble water dam with beveled front and back edges.  Miter threshold at corners (if needed).  See plan for water dam length(s) and orientation.  Straight runs of threshold shall be single pieces (no splices).Shower Threshold:  Roman Marble Products, Inc., at www.counterstone.org or 800-347-6699 or approved substitute.

Shower Curb:  Install a cultured marble custom curb.  See plan for length of curb needed.  Straight runs of curb shall be single pieces (no splices).  Curb to be cut to maximum height of 3” AFF (bathroom floor).  Curb to be modified according to manufacturer’s instructions or ordered cut to size. Manufacturer prefers to consult with installer on modification prior to installation. Curb to be adhered with 100% silicone adhesive.

Shower Curb:  Roman Marble Products, Inc., at www.counterstone.org or 800-347-6699.

New Hand Held Shower (Grab Bar Mount):  Install a new hand-held showerhead with pushbutton pause, 70" showerhead hose, an adjustable height mounting bracket, a 24"/36” vertically oriented grab bar (for showerhead mount - provide blocking as needed to mount for regular grab bar use), and miscellaneous fittings and accessories as needed (field verify).  Coordinate with consumer/family for mounting location prior to installation. Handheld Showerhead with Grab Bar Support:  Delta Single Function Personal Handheld Shower with 24” Grab Bar Mount and 69” Hose (Item #RPW324HDF), at www.build.com or 800-375-3403 or deltafaucet.com; Delta Adjustable Grab Bar Shower with 24” Grab Bar and 70” Uniflex Hose (Item #52001UF); Alsons Single Function Handheld Shower with 36” Grab Bar Mount and 69” Hose (Item #52003 DS), at www.build.com or 800-375-3403 or approved substitute.  Delta stretchable 69”-92” metal hand shower hose #U495D-69-PK at deltafaucet.com.  Delta adjustable shower arm mount Model #U3401-PK at deltafaucet.com.  Delta 1-Spray hand shower Model #59462-WHB-PK at deltafaucet.com

Delta Strechable 69” to 92” 			Delta Adj. Shower 		Delta 1-Spray Hand

Metal Hand Shower Hose			Arm Mount			Shower

Showerhead Mount on Grab Bar: Delta Grab Bar Handheld Showerhead Mount (on additional grab bar); Moen Slide Bar Bracket (Item #52711), at www.build.com or 800-375-3403; Moen Grab Bar Shower Holder (Item #DN8005), at www.homedepot.com or 800-925-6278 or approved substitute.

New Hand Held Shower (Glide Rail):  Provide and install a new hand-held showerhead with pushbutton pause, 70" showerhead hose, an adjustable height mounting bracket, a 24" vertically oriented hose nut wall bar, and miscellaneous fittings and accessories as needed (field verify).  Coordinate with consumer/family for mounting location prior to installation.

Handheld Showerhead with Slide Rail: Delta Single Function Personal Hand Shower with 24” Slide Bar and 69” Hose (Model #RPW324HDF-1.5), at www.build.com or 800-375-3403; Isola hand shower with slide bar (Model #922512), at www.signaturehardware.com or 866-855-2284

Hand Held Shower			Isola Hand Shower

(Glide Rail)

New Hand Held Shower (Tub/Shower Combo):  Provide and install a new hand-held showerhead with pushbutton pause, 70" showerhead hose, a pin wall mount with swivel connector, diverter tub spout, and miscellaneous fittings and accessories as needed (field verify).

Showerhead and Mounting System:  Alsons Four Spray Wall Mount Hand Shower Unit (Item #5445), at www.plumberssurplus.com or 888-859-2080; Price Pfister Pasadena Handheld Shower, Single Handle Tub and Shower Faucet (Model #8P8-PDHK), at www.plumberssurplus.com or 888-859-2080 or approved substitute.

Shower Mount:

Provide and install a new Mommy’s Helper Safer Grip Portable Shower Arm for use in shower with existing hand held shower system.  Coordinate best position for installation with family.  Mommy’s Helper Safer Grip Portable Shower Arm available on www.amazon.com.

Tub/Shower Faucet: Install a light commercial grade ADA compliant tub/shower single lever handle, pressure-balanced shower faucet with temperature limit stops, shower arm, 2" steel drain, diverter tub spout, plumbing supplies and accessories as needed.  Install faucet and plumbing including hot and cold water supply and drain, per codes and per manufacturer’s recommendations.

Shower Faucet: Moen Chateau (Model #L2352), at www.moen.com or 800-289-6636; American Standard Colony Soft (Model #T675507), at www.americanstandard-us.com or 1-800-442-1902 or approved substitute.

Tub/Shower Faucet:  Moen Chateau (Model #L2353), at www.moen.com or 800-289-6636; American Standard Colony Soft (Model #T675508), at www.americanstandard-us.com or 1-800-442-1902 or approved substitute.

Folding Shower Seat:  Install a wall mounted shower seat with fold-away legs and fold-up features with weight capacity of 285 pounds.  Adequate wall blocking must be provided and seat installed per manufacturer’s recommendations.  Seat covering to be molded plastic, wood, or vinyl (coordinate with consumer/family) and must be mold-resistant.  Seat to be installed at 18” AFF/Coordinate with consumer/family on desired height/location of seat. Folding Shower Seat: Etac Rufus Wall Mounted Shower Seat (Model #557401, 17” W X 18” D”, at www.pattersonmedical.com or 800-343-9742 or approved substitute.

Heavy Duty Folding Shower Seat:  Install a folding 4-legged shower bench with no wall reinforcement needed. Legs to individually adjust for correct seat height and level surface.  Bench to also include phenolic top to ensure easy maintenance.  Bench shall be right or left, L-shaped version. Shower transfer bench shall measure approximately 32"W x 21"D x 15"D and adjusts to 17"-19" in height with a weight capacity of 1000 lbs.  Install per manufacturer’s recommendations.

Heavy Duty Folding Shower Seat: Folding Shower Transfer Bench (Model #920501 (Right) or Model #920502 (Left)), at www.pattersonmedical.com or 800-343-9742 or approved substitute.

End Mount w/Swing Down Legs Shower Seat:  Install one end mounted shower seat with swing down adjustable legs. Frame of seat to be made of 1”x1” tube stainless steel. Swing down legs to automatically lock when seat is lowered. The seat will automatically lock in the “up” position. Seat covering to be teak, phenolic or white finish.

End Mounted w/Swing Down Legs Shower Seat:  Portable Tub Seat (with adjustable clamp) (Model #STPA-265150-PTS, 26 1/2”L x 15”W, Adjustable Leg Height, 12 3/4” to 14”), at www.seachrome.com or 800-955-2476 or approved substitute.

Miscellaneous Shower Accessories: Install an economy grade vinyl window curtain at window, cut to fit.  Install a mildew resistant shower curtain with weighted bottom, shower curtain rod/L-shaped shower curtain rod with corner ceiling suspension rod at corner/crescent shower curtain rod, and miscellaneous shower curtain/rod accessories. Verify length and height of shower curtain rod.  Secure shower curtain rod with flanges screwed to both walls above bathtub/shower entry ensuring that shower curtain touches the floor and approximately 1” inside shower threshold.  Reinstall miscellaneous (previously removed) bathroom accessories in appropriate location and height as needed.

Weighted Shower Curtain: Antibacterial Weighted Shower Curtain (48”/71” x 74”), at www.aquabath.com or 800-232-2284; ADA Shower Curtains (48”/72”x74”), at www.inprocorp.com or 800-222-5556 or approved substitute.

Corner Shower Curtains Rod: Barclay Products 60” Corner Rod with Flanges in Polished Chrome (Model #4121-60-CP, Internet # 202496895), at www.homedepot.com;  Corner Shower Curtain Rod with Sleeve Ceiling Support or Corner Curtain with J Support (Model # Q-3060/C-7 or 3060/C-8), at www.calltsm.com or 800-472-2227 or approved substitute.

Crescent Shower Curtain Rod: Fasten shower curtain rod (60”/72”) securely at both ends of rod as per manufacturers recommendations.  WingIts Original Crescent Curved 5’0” Shower Curtain, at www.wingits.com or 877-894-6448; Zenith Single Curved Shower Rod in Brushed Nickel (Model #35603BN06, Internet # 202061596), at www.homedepot.com or approved substitute.

Caregiver Curtain:  Install a caregiver curtain at a height appropriate for caregivers to provide bathing assistance from outside of the shower.  The rod shall be telescoping, lift out or retractable.  Install per manufacturer’s recommendations.

Telescoping/Swing Away Curtain Rod: This rod will extend from a range of 39” to 91” and the rod will swing clear of the shower area allowing consumer to enter freely. Telescoping Curtain, (Model # PST/ANTI) at www.rbwire.com or 800-634-0555; Telescoping Pivoting Shower Rod, (Model #300B-ACXCURT0010) at www.store.adaptivelivingstore.com or 877-447-7816 or approved substitute. Removable Rod:  Curtain Rod, at www.best-bath.com or 800-727-9907 or approved substitute.

Ceiling Mounted Exhaust Fan:  Install a ceiling mounted exhaust fan.  Vent to outside to a back flow preventing louvered grill.  Wire to separate switch near existing main light fixture switch.  Wire to same switch as ceiling light fixture. Exhaust Fan:  Nutone 80 CFM Ceiling Mount Exhaust Bath Fan, Model #684NT, Internet # 100066718), at www.homedepot.com.

Ceiling Mounted Exhaust Fan/Light Fixture:  Install a ceiling mounted exhaust fan/light fixture unit. Vent to outside to a back flow preventing louvered grill.  Wire to separate switch near existing main light fixture switch.  Wire to same switch as ceiling light fixture. Exhaust Fan/Light:  Nutone 70 CFM Ceiling Exhaust Fan/Fluorescent Light, Model #769RFT, Internet # 10049031), at www.homedepot.com.

Ceiling Mounted Exhaust Fan/Light Fixture with Heat:  Install a ceiling mounted exhaust fan/heat/light fixture unit.  Provide and install a three-function control.  Vent to outside to a back flow preventing louvered grill. Wire to separate switch near existing main light fixture switch.  Wire to same switch as ceiling light fixture. Exhaust Fan/Light/Heater:  Nutone 70 CFM Ceiling Exhaust Fan with Light and Heater (1300 watt heater) (Model #665RP, Internet # 100198855), at www.homedepot.com.

Ceiling Light Fixture:  Install an economy grade ceiling mounted light fixture with a 12"x12" translucent diffuser and sockets for 2 standard incandescent light bulbs (consult with consumer for style/finish). Wire to separate switch near existing main light fixture switch.  Wire to same switch as ceiling light fixture.

Vanity Light Fixture:  Install an economy grade vanity light fixture (consult with consumer for style/finish). Wire to separate switch near existing main light fixture switch.  Wire to same switch as ceiling light fixture.

Medicine Cabinet:  Install an economy grade medicine cabinet (consult with consumer for style/finish).  Door shall swing towards adjacent wall to offer clear access to the cabinet interior.  Mirror shall be mounted with bottom @ 40" A.F.F. unless otherwise noted or consult with consumer/family prior to installation.

Wall Mounted Mirror:  Install a wall mounted frameless 24" x 36" mirror.  Mirror shall be mounted with bottom 40" A.F.F. unless otherwise noted or consult with consumer/family prior to installation.

Swivel Wall Mounted Mirror:  Install an adjustable mirror that can be adjusted up to 30 degrees. Mirror must have an easy control to adjust mirror.

Swivel Wall Mounted Mirror: Gatco (31 1/2”H x 23 1/2”W) Tiara Rectangular Frameless Bath Mirror with Beveled Edges (Model #4339S), at www.lowes.com; Vari-Tilt M-3 (18”H 24”W), at Tubular Specialties Manufacturing, www.calltsm.com or 800-225-5876 or approved substitute.

Wall Mounted Register Heater:  Install wall mounted heater in bathroom wall.  eater to be made of 24-guage steel construction and mount between wall studs.  Unit to be 1500W at 120 VAC.  Heat control for wall unit to be handled by a wall thermostat.

Wall Mounted Register Heater:  Broan (Model 174 or Model 84, Exterior Surface Mounting Kit), at www.broan.com or 800-558-1711 or approved substitute.

Washer/Dryer Hookups:  Provide and install hookups (plumbing, gas, electrical, and exterior vents) as needed to relocate existing washer and dryer from basement to room at top of basement steps as shown on drawing.  Field verify the hookup locations with consumer/family.  Include a standard consumer-grade single handle quarter turn dual shut off valve for the water supply to the washer mounted in a prefabricated valve and drain box.  Recess valve and drain box into wall.  Use backflow-preventing mechanism in dryer vent.  Provide and install washer pan for washer to sit in.  Move and install washer and dryer into current location and connect to hookups providing miscellaneous items to complete installation (ex. Dryer vent hose)

Tubcut Installation:  (To be completed by Tubcut contractor)

Provide and install a XX” TubcuT in a bathtub (metal/fiberglass tub application) per manufacturer’s recommendations.  Field verify and discuss with family on best location of cut - consider making majority of opening at front/back section of tub. Salvage piece to family.  TubcuT information available at www.tubcut.com.

Safeway Safety Step Installation:  (To be completed by Safeway Safety Step contractor)

Provide and install a 24” Safeway Safety Step in a bathtub (metal/fiberglass tub application) per manufacturer’s recommendations.  Field verify and discuss with family on best location of cut - consider making majority of opening at front/back section of tub (towards plumbing wall).  Safeway Safety Step information available at www.safewaystep.com.  Adjust shower curtain/rod as needed.